# QR Codes and Barcodes

# 1. QR Codes: Mechanics and Functionality

## What is a QR Code?

A Quick Response (QR) code is a two-dimensional (2D) matrix barcode consisting of black and white squares arranged in a grid on a square background. Invented in 1994 by Denso Wave, it was originally designed for tracking automotive parts.

## How does it work?

- Structure: It features "position detection markers" (three large squares in the corners) that help scanners determine the orientation and size of the code.

- Encoding: Data is encoded both horizontally and vertically. It uses four standardized modes: numeric, alphanumeric, byte/binary, and Kanji.

- Error Correction: QR codes use Reed-Solomon error correction, allowing the code to be read even if up to 30% of it is damaged or obscured.

- Scanning: It is scanned using imaging devices (like smartphone cameras) which convert the patterns of squares into binary data.

# 2. Barcodes: Mechanics and Functionality

## What is a Barcode?

A traditional barcode (1D) is a machine-readable optical image that represents data using parallel lines of varying widths and spacings. The most common version is the UPC (Universal Product Code) found on retail items.

## How does it work?

- Structure: It consists of a series of black bars and white spaces. A "quiet zone" (white space) is required at both ends to help the scanner identify where the code starts and stops.

- Encoding: Data is stored in one direction (horizontally). Each pattern of bars represents a specific character or number.

- Scanning: Traditionally scanned using laser scanners that measure the light reflected off the bars and spaces. Modern "imager" scanners can also read them using cameras.

# 3. Key Similarities and Differences

| Feature | Barcode (1D) | QR Code (2D) |
| --- | --- | --- |
| Data Direction | One-dimensional (Horizontal) | Two-dimensional (Horizontal & Vertical) |
| Data Capacity | Low (typically 20-25 characters) | High (up to 7,089 numeric characters) |
| Data Types | Alphanumeric only | URLs, multimedia, binary, text, etc. |
| Error Tolerance | Low (damage usually breaks it) | High (readable even if partially damaged) |
| Scanning Speed | Extremely fast for simple lookups | Slightly slower due to data complexity |
| Size | Requires length to store more data | Can be very small and still hold high data |
| Customization | Very limited (mostly B&W lines) | High (can include colors, logos, icons) |

# 4. Advantages and Disadvantages for Augmented Reality (AR)

## QR Codes for AR

- Advantages:

  - Data Richness: Can store a URL that directly launches a web-based AR experience (WebAR) without a dedicated app.

  - 360° Scanning: Can be scanned from any angle, which is crucial for users moving around a physical object.

  - Robustness: High error correction means it works well in the real world where lighting or physical damage might occur.

  - Marker Tracking: The distinct corner markers make them excellent "anchors" for the AR engine to place 3D objects accurately.

- Disadvantages:

  - Aesthetics: Some designers find the "pixelated" look unattractive compared to clean logos or graphics.

## Barcodes for AR

- Advantages:

  - Ubiquity: Almost every consumer product already has a barcode, allowing for "retroactive" AR experiences (e.g., scanning a cereal box to see a 3D game).

- Disadvantages:

  - Directional Sensitivity: Harder for AR engines to track because they must be viewed from specific angles to be "decoded."

  - No Native Action: Usually requires a middle-man app to look up the barcode in a database before triggering an AR effect.

  - Lack of Depth: Does not provide as many unique tracking points for stable 3D object placement.

# 5. Which is more suitable for AR?

The QR code is significantly more suitable for AR.

Why?

- Fiducial Marking: The "Finder Patterns" (the squares in the corners) serve as perfect fiducial markers. This allows the AR software to calculate the exact distance, tilt, and rotation of the camera relative to the code, resulting in a stable 3D object placement.

- Instant Accessibility: Because a QR code can store a full URL, it can bypass the friction of app downloads. A user simply points their camera, taps a link, and the AR experience begins.

- Stability: The 2D grid provides more "tracking points" for the computer vision algorithms than the simple parallel lines of a barcode.